OLYMPIC COMPRESSION VALUES

Shire 1950 ... 4TNE84

| Cylinder | PSI | BAR | Temp |
| --- | --- | --- | --- |
| No 1 | 330 | 23 | COLD |
| No 2 | 230 | 16 | COLD |
| No 3 | 290 | 20 | COLD |
| No 4 | 220 | 15 | COLD |
| xxxxxxxxxx | xxxxxxxxxx | xxxxxxxxxx | xxxxxxxxxx |
| No 1 | 360 | 25 | HOT |
| No 2 | 310 | 21 | HOT |
| No 3 | 310 | 21 | HOT |
| No 4 | 290 | 20 | HOT |

1 kilogram-force/square centimeter = 14.223 343 334 pound/square inch

1 kilogram-force/square centimeter = 0.980 665 bar

Our Engine is a 4TNE84 and should be:-		33 +/– 1 Kg/sqcm 	 Standard

25 Kg/sqcm 		 Limit

Permitted variation between cylinders 		2 – 3 Kg/sqcm

Required for Cylinder Head Overhaul

2 x Big Stilson Wrenches

Valve Spring Compressor		Grinding Paste 	 Valve grinding Tool

Socket Set				Screwdrivers

Torque Wrenches			Feeler Guages

Work Mate

Generator				Power Drill		Wire Brushes

Gaskets				Red Hermetite

Rags					Cleaning Fluid (Spray Can)

Workshop Manual			Compression Values

Kitchen Roll

Crimp Terminals

Boiler Suit	Work Shoes	Hat	Gloves

Waterproof

Tea brewing stuff