# PROPELLER SHAFT & PROPELLER DETAILS

Current Arrangement

A Vetus 35mm propshaft with standard taper, shortened by about 9" at the driven (untapered) end.

A Vetus 19" propeller. Profile: P3B (for boats up to 10Km/hour), Right handed. (was LH)

Our Morse linkages are arranged to run the gear box so forwards means forwards. The Barrus gear-box spec is symmetric - it doesn't care which way it rotates.

This is all standard "off the shelf" equipment from a Vetus supplier (Vetus is the only metric equipment). Note that the stern tube is a 35mm Vetus part so "inch and a quarter / half" parts don't fit.

In the NW, Vetus suppliers are at Kings Lock (Middlewich), Garstang and Leeds. I (PhilB) have a full parts catalogue.

Original Arrangement as Fitted by Pipers.

What we had was non-standard and modified parts.

A Vetus 35mm propshaft with the taper machined down to a (now) known but unidentified profile (which doesn't fit the even older imperial 18" prop I have under my floor).

An unknown prop, probably 18" but left-handed to suit to the old Lister (non-reversible) rotation. Left-handed props are uncommon and not stocked.

A Woodruff key to fit the propshaft which had been crudely narrowed to fit the smaller keyway of the prop. This (amazingly) stayed in the propshaft. It was almost certainly a sloppy fit in the prop (since the remaining up-stand is very narrow) and would probably have allowed a slight movement (rotation) of the prop on the shaft which in time could loosen the nut.

No locking washer - once loosened the nut could spin off (and did!).

In store I have this prop shaft and the previous 18” propeller which was fitted prior to the redesign by Pipers at the time of over-plating.